CMPN301 – Computer Architecture

Project Phase 1 Report

| Team Members |  |
| --- | --- |
| Farah Mohamed Abdelfattah | 1190176 |
| Abdelrahman Mohamed Ezzat | 1190158 |
| Ziad Ahmed Hamed | 4200002 |
| Ali Mohamed Hashish | 1190223 |
| Tutorial | Thursday 8 am |

| OPCODES FOR EACH INSTRUCTION |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NOT | 1 | 1 | 0 | 0 | 0 |  |
| INC | 1 | 1 | 0 | 0 | 1 |  |
| DEC | 1 | 1 | 0 | 1 | 0 |  |
| MOV | 1 | 1 | 0 | 1 | 1 |  |
| ADD | 1 | 1 | 1 | 0 | 0 |  |
| IADD | 1 | 1 | 1 | 0 | 0 | 1 |
| SUB | 1 | 1 | 1 | 0 | 1 |  |
| AND | 1 | 1 | 1 | 1 | 0 |  |
| OR | 1 | 1 | 1 | 1 | 1 |  |
| PUSH | 0 | 1 | 0 | 0 | 0 |  |
| POP | 0 | 1 | 0 | 0 | 1 |  |
| LDM | 0 | 1 | 0 | 1 | 0 | 1 |
| LDD | 0 | 1 | 0 | 1 | 1 |  |
| STD | 0 | 1 | 1 | 0 | 0 |  |
| JZ | 1 | 0 | 0 | 0 | 0 |  |
| JC | 1 | 0 | 0 | 0 | 1 |  |
| JMP | 1 | 0 | 0 | 1 | 0 |  |
| CALL | 1 | 0 | 0 | 1 | 1 |  |
| RET | 1 | 0 | 1 | 0 | 0 |  |
| RTI | 1 | 0 | 1 | 0 | 1 |  |
| NOP | 0 | 0 | 0 | 0 | 0 |  |
| SETC | 0 | 0 | 0 | 0 | 1 |  |
| CLRC | 0 | 0 | 0 | 1 | 0 |  |
| IN | 0 | 0 | 0 | 1 | 1 |  |
| OUT | 0 | 0 | 1 | 0 | 0 |  |

| INSTRUCTION FORMAT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| OPCODE |  |  |  |  | Immediate bit | R destination |  |  | R source 1 |  |  | R source 2 |  |  | unused |  |

| CONTROL SIGNALS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RegWrite | PCSrc | MemRead (used by memory) | MemWrite (used by memory) | MemToReg (used by WB) | IN (used by WB) | OUT (used by WB) | SP+ (used by memory) | SP- (used by memory) |
| NOT | 1 |  |  |  |  |  |  |  |  |
| INC |  |  |  |  |  |  |  |  |  |
| DEC |  |  |  |  |  |  |  |  |  |
| MOV |  |  |  |  |  |  |  |  |  |
| ADD |  |  |  |  |  |  |  |  |  |
| IADD |  |  |  |  |  |  |  |  |  |
| SUB |  |  |  |  |  |  |  |  |  |
| AND |  |  |  |  |  |  |  |  |  |
| OR |  |  |  |  |  |  |  |  |  |
| PUSH |  |  |  | 1 |  |  |  |  | 1 |
| POP | 1 |  | 1 |  | 1 |  |  | 1 |  |
| LDM | 1 |  |  |  |  |  |  |  |  |
| LDD | 1 |  | 1 |  | 1 |  |  |  |  |
| STD |  |  |  | 1 |  |  |  |  |  |
| JZ |  | 1 |  |  |  |  |  |  |  |
| JC |  |  |  |  |  |  |  |  |  |
| JMP |  |  |  |  |  |  |  |  |  |
| CALL |  |  |  | 1 |  |  |  |  | 1 |
| RET |  |  | 1 |  |  |  |  | 1 |  |
| RTI |  |  | 1 |  |  |  |  | 1 |  |
| NOP |  |  |  |  |  |  |  |  |  |
| SETC |  |  |  |  |  |  |  |  |  |
| CLRC |  |  |  |  |  |  |  |  |  |
| IN | 1 |  |  |  |  | 1 |  |  |  |
| OUT |  |  |  |  |  |  | 1 |  |  |

| ALL HAZARDS |  |
| --- | --- |
| Hazards | Details |
| Data Hazards | Forwarding unit at ALU: Checks for RegWrite signal at all subsequent buffers Checks for Rd for all subsequent buffers Checks for Rs1,Rs2 of current execution And acts accordingly -> checks closer buffers first Potential instructions to cause hazards [All R-type, POP, IN, LDD, LDM] Forwarding unit at Memory: Checks for RegWrite signal at MEM2/WB buffer Checks for Rd at MEM2/WB buffer Checks for Rs1, Rs2 of current STORE operation And acts accordingly Potential instructions to cause hazards [a load (LDD) instruction followed by a store (STD)] |
|  | Hazard Detection unit at Decode: Checks for MemRead (only by LDD and POP) signal at ID/EX buffer Checks for Rd at ID/EX buffer Checks for Rs1, Rs2 of current fetched instruction (from IF/ID buffer) And acts accordingly Potential instructions to cause hazards [a load (LDD) instruction or a pop (POP)] |
| Structural Hazards | Hazard Detection unit at Decode: If current MemRead or MemWrite at decode stage are ‘1’ Checks for MemRead or MemWrite at ID/EX buffer if equal ‘1’ Then stall once |
| Control Hazards | Static Branch Prediction: Predict untaken, If taken: JMP [decode] Change PC -> to RD Flush IF/ID buffer (1 Stall) Call [decode] Change PC -> to RD Flush IF/ID buffer Propagate old PC till Mem stage to save it |
|  | JMP conditional [execute] (JZ – JC) Change PC -> to RD Flush IF/ID & ID/EX (2 stalls) |
|  | RET/RTI [memory] Change PC -> to DataMem[SP] Flush IF/ID & ID/EX & EX/Mem1 & Mem1/Mem2 (5 stalls) |

| PIPELINE REGISTERS |  |  |  |
| --- | --- | --- | --- |
| Register | Inputs | Outputs |  |
| IF/ID | - instruction[15..0] - immediate[15..0] - PC[15..0] | - RS1[2..0] - RS2[2..0] - instructionOut[15..0] - immediateOut[15..0] - PCout[15..0] |  |
| ID/EX | - Clk - inPort - memRead - memToReg - memWrite - outport - pcSrc - regWrite - reset - spDec - spInc - RS1data[15..0] - RS2data[15..0] - instruction[15..0] - immediate[15..0] - PC[15..0] | - inPortOut - isImmediate - memReadOut - memToRegOut - memWriteOut - outPortOut - regWriteOut - spDecOut - spIncOut - RS1dataOut[15..0] - RS2dataOut[15..0] - opcode[4..0] - RD[2..0] - immediateOut[15..0] - PCout[15..0] |  |
| EX/MEM1 | - Clk - inPort - memRead - memToReg - memWrite - outport - regWrite - reset - spDec - spInc - RDreg[2..0] - RS1data[15..0] - RS2data[15..0] - ALUresult[15..0] - PC [15..0] | - inPortOut - memReadOut - memToRegOut - memWriteOut - outPortOut - regWriteOut - spDecOut - spIncOut - RDregOut[2..0] - RS1dataOut[15..0] - RS2dataOut[15..0] - ALUresultOut[15..0] |  |
| MEM1/MEM2 | - dataMemOutput[15..0] - RS1Data[15..0] - ALUoutput[15..0] - RD[2..0] - regWrite_in - MemToReg_in - INN_in - OUTT_in | - dataMemOutput[15..0] - RS1Data[15..0] - ALUoutput[15..0] - RD[2..0] - regWrite_in - MemToReg_in - INN_in - OUTT_in |  |
| MEM2/WB | - dataMemOutput[15..0] - RS1Data[15..0] - ALUoutput[15..0] - RD[2..0] - regWrite_in - MemToReg_in - INN_in - OUTT_in | - dataMemOutput[15..0] - RS1Data[15..0] - ALUoutput[15..0] - RD[2..0] - regWrite_in - MemToReg_in - INN_in - OUTT_in |

| BUFFER INPUTS (FROM QUARTUS – WITH SIZES) |  |
| --- | --- |
| IF/ID buffer | Memory stage - 2 buffers are inside |
| ID/EX buffer | EX/MEM1 buffer |

SCHEMATIC OF THE DESIGN

TODOS:

- PCSrc mux must be a unit

- Change MEM[0] “initial PC” & MEM[1] “interrupt” to be in Instruction Cache

- Jump already fetched then an interrupt happens [code]

- In data propagated through all pipeline registers [Ziad]

- In signal stops at ALU & overwrite ALU output [Ziad]

- Load use case when stalling only once [MEM1/MEM2 buffer]

- Reading flags from stack ->  reading 32 bits

- Modify increments to PC in design

Tasks:

- Modify design

- Check XXX flags

- Expand instruction cache size [ask about expected size]

- Data hazards [HDU] [Farah]

- Structural hazards [HDU] [Hashish]

- Control hazards [Ziad and Boody]

- Interrupts [Hashish]

- Forwarding units

https://app.diagrams.net/#G14Fvc_gAKcYsJDFnu-0WdP1k0BAtX3QD7